# ASSESSMENT TOOL 3
COMPUTER VISION WITH OPENCV

# Question 1: Image Preprocessing and Feature Extraction

## (a) Preprocessing Pipeline

Noise Reduction: Apply Gaussian or Median Filter to remove sensor noise while preserving crop details.

Illumination Correction: Use Histogram Equalization or CLAHE to reduce shadows and uneven sunlight.

Contrast Enhancement: Enhance crop visibility using CLAHE.

Normalization: Normalize pixel values (0–1 or 0–255) for consistent processing.

Pipeline:
Captured Image
   ↓
Noise Reduction
   ↓
Illumination Correction
   ↓
Contrast Enhancement
   ↓
Normalization
   ↓
Preprocessed Image

## (b) Feature Extraction

Recommended Method: Local Binary Pattern (LBP).

Reasons: Detects texture changes, robust to illumination changes, computationally efficient, suitable for pest-damaged crop detection.

Diagram:
Healthy Crop: ████████
Pest Crop: █▓▒░▓██▒░
LBP extracts texture differences.

## (c) Rotation and Scale Variations

Rotation and scale variations affect feature matching because crops appear at different sizes and orientations.

Use SIFT, SURF, or ORB to achieve scale- and rotation-invariant feature detection.

# Question 2: Stereo Vision and Motion Estimation

## (a) Stereo Disparity and Depth Estimation

Stereo cameras capture left and right views. The horizontal difference between matching points is called disparity.

Depth = (Focal Length × Baseline) / Disparity

Nearby objects have high disparity and smaller depth values, while distant objects have low disparity and larger depth values.

Diagram:
Left Camera ●-----------● Right Camera
        Car
Large disparity = Near
Small disparity = Far

## (b) Optical Flow for Motion Tracking

Optical flow estimates pixel movement between consecutive frames to track pedestrians and vehicles.

Algorithms: Lucas–Kanade and Horn–Schunck.

Diagram:
Frame 1: ●
Frame 2:     ● →

## (c) Errors and Mitigation Techniques

| Source of Error | Mitigation |
| --- | --- |
| Camera Misalignment | Calibration and Rectification |
| Poor Lighting | Histogram Equalization / CLAHE |
| Occlusion | Multi-frame tracking |
| Fast Motion | High frame-rate cameras |
| Textureless Regions | SIFT / ORB feature matching |

Workflow:
Stereo Cameras
 ↓
Calibration
 ↓
Disparity Map
 ↓
Depth Estimation
 ↓
Optical Flow
 ↓
Collision Avoidance

# Conclusion

Image preprocessing improves image quality for accurate analysis. LBP detects pest-related texture changes, while SIFT/SURF/ORB handle rotation and scale variations. Stereo vision estimates depth using disparity, and optical flow tracks moving objects. Calibration and robust preprocessing improve overall system performance.